12V to 5V DC-DC Buck Converter

PCB Design Report

Designed in KiCad — Schematic, PCB Layout, Validation & Fabrication Package

Author: [S.Simanchal Rao]

Date: June 2026

# Table of Contents

Table of Contents	1

1. Introduction	1

1.1 Project Objectives	1

2. System Overview	1

3. Schematic Design	1

3.1 Core Components	1

3.2 Design Calculations	1

4. PCB Layout	1

4.1 Layer Stack-Up	1

4.2 Layout Practices Applied	1

4.3 Mechanical Details	1

5. Design Validation	1

5.1 Electrical Rule Check (ERC)	1

5.2 Design Rule Check (DRC)	1

6. Fabrication Outputs	1

6.1 Bill of Materials (Summary)	1

7. Conclusion	1

7.1 Future Improvements	1

7.2 References	1

# 1. Introduction

This report documents the complete design process of a 12V to 5V DC-DC buck converter, from schematic capture through to a fabrication-ready 2-layer printed circuit board (PCB). The project was carried out using KiCad, an open-source electronics design automation (EDA) suite, covering schematic design, component selection, PCB layout, design validation, and generation of manufacturing outputs.

The objective of this project was to design a reliable, efficient step-down power supply capable of converting a 12V DC source (such as a wall adapter or battery pack) into a regulated 5V DC rail suitable for powering microcontrollers, sensors, and other low-voltage digital electronics.

## 1.1 Project Objectives

- Design a stable, regulated 5V output from a 12V DC input using a switching buck topology

- Select all power components (regulator IC, inductor, capacitors, diode) based on manufacturer datasheet specifications

- Lay out a compact, low-noise 2-layer PCB following accepted power-electronics design practices

- Validate the design electrically and physically before committing to fabrication

- Produce a complete, manufacturer-ready fabrication package (Gerbers, drill files, BOM)

# 2. System Overview

A buck converter is a switch-mode power supply (SMPS) topology that steps down a DC voltage efficiently, typically achieving 75-90%+ efficiency depending on load and component selection, compared to the much lower efficiency of a linear regulator performing the same conversion (which would dissipate the voltage difference as heat).

The key design parameters for this project are summarized below.

| Parameter | Specification |
| --- | --- |
| Input Voltage (Vin) | 12V DC (nominal) |
| Output Voltage (Vout) | 5V DC (fixed) |
| Maximum Output Current | 3A |
| Topology | Non-synchronous buck (step-down) |
| Switching Frequency | 150 kHz (fixed internal oscillator) |
| PCB | 2-layer, through-hole component design |

# 3. Schematic Design

The schematic was captured in KiCad's Schematic Editor (Eeschema), following the standard fixed-output application circuit recommended by the regulator manufacturer. The core of the design is the LM2596S-5.0, a monolithic step-down switching regulator with a fixed 5V output, integrated oscillator, and internal power switch rated for up to 3A of output current.

## 3.1 Core Components

| Ref | Component | Selected Part | Function |
| --- | --- | --- | --- |
| U1 | Switching Regulator | LM2596S-5.0 (TO-263-5) | Fixed 5V, 3A step-down switching regulator with internal power switch |
| L1 | Power Inductor | 68 µH, radial leaded | Energy storage element; sets ripple current at the chosen switching frequency |
| D1 | Catch Diode | 1N5822 Schottky, 3A / 40V | Freewheeling diode; provides current path when switch is off |
| C1 | Input Capacitor | 100 µF / 25V electrolytic | Bulk decoupling at the regulator input |
| C2 | Output Capacitor | 220 µF / 25V, low-ESR electrolytic | Output filtering; minimizes output ripple voltage |

## 3.2 Design Calculations

Component values were derived using the design procedure and selection charts provided in the LM2596 datasheet, rather than calculated purely from first-principles inductor/capacitor design equations. This is the manufacturer-recommended approach for this family of regulators, since the datasheet charts already account for the IC's internal switch characteristics and current limits.

Duty Cycle Estimate

For an ideal buck converter, the approximate duty cycle is given by:

D = Vout / Vin = 5V / 12V ≈ 0.417  (41.7%)

This indicates the regulator's internal switch conducts for approximately 41.7% of each switching cycle under steady-state conditions, with some additional duty cycle added to compensate for diode and switch voltage drops.

Inductor Selection

The LM2596 datasheet provides an inductor selection chart plotted against output current and the Vin/Vout ratio. For Vin = 12V, Vout = 5V, and Iout(max) = 3A, the chart indicates an inductance value of 68 µH (commercially available as a standard value, e.g. Bourns RLB0914 series). This value balances ripple current against physical inductor size and saturation current rating.

Output Capacitor Selection

A low-ESR electrolytic capacitor was selected for the output (220 µF / 25V) per the datasheet's recommended capacitor table. Low ESR is important here because output ripple voltage in a buck converter is dominated by the capacitor's equivalent series resistance rather than its capacitance value alone at this switching frequency.

Input Capacitor and Catch Diode

A 100 µF / 25V electrolytic capacitor was placed at the input to absorb the pulsed input current drawn by the regulator's internal switch. The 1N5822 Schottky diode was selected as the catch diode because its reverse voltage rating (40V) comfortably exceeds the input voltage, its forward current rating (3A) matches the maximum output current, and its low forward voltage drop reduces conduction losses compared to a standard PN junction diode.

# 4. PCB Layout

The PCB was designed as a 2-layer board in KiCad's PCB Editor (Pcbnew). Layout decisions were guided by standard switch-mode power supply design practices to minimize electrical noise, voltage ripple, and electromagnetic interference (EMI).

## 4.1 Layer Stack-Up

| Layer | Purpose |
| --- | --- |
| Top Copper (F.Cu) | Component placement and primary signal/power routing |
| Bottom Copper (B.Cu) | Dedicated ground plane for low-impedance return path |

## 4.2 Layout Practices Applied

- Ground plane: a continuous ground plane was used on the bottom layer to minimize return-path impedance and reduce ground bounce

- Short, wide switching loop: the high-current loop formed by the regulator's switch pin, inductor, output capacitor, and ground return was kept as short and wide as possible to minimize loop inductance and radiated EMI

- Capacitor placement: input capacitor (C1) was placed directly adjacent to the regulator's Vin pin, and the output capacitor (C2) was placed close to the inductor and output connector

- Trace widths: power traces were sized to safely carry the 3A maximum output current with acceptable temperature rise, using standard PCB trace-width/current-capacity references

- Thermal management: copper area near the regulator IC was increased to aid heat dissipation from the TO-263 package's exposed tab

- Net separation: the feedback/control path was routed away from the high-current switching loop to avoid noise injection

## 4.3 Mechanical Details

| Property | Value |
| --- | --- |
| Board dimensions | [Fill in actual dimensions, e.g. 50mm x 40mm] |
| Layer count | 2 |
| Copper weight | 1 oz (35 µm), standard |
| Mounting | [Fill in, e.g. 4x M3 mounting holes] |
| Connectors | 2-pin terminal blocks for input (J1) and output (J2) |

# 5. Design Validation

Before generating manufacturing files, the design was validated using KiCad's built-in checking tools to catch electrical and physical errors early, reducing the risk of a non-functional or unmanufacturable board.

## 5.1 Electrical Rule Check (ERC)

The ERC was run in the Schematic Editor to check for common schematic-level errors such as unconnected pins, conflicting net types, missing power flags, and duplicate reference designators.

- Verified all power input pins (Vin, GND) were properly connected to defined power nets

- Confirmed no unconnected pins remained on the regulator IC, diode, or passive components

- Resolved all warnings related to power flags on the input/output net labels

Result: No outstanding ERC errors or warnings.

## 5.2 Design Rule Check (DRC)

The DRC was run in the PCB Editor to verify the physical layout against manufacturing constraints, including copper clearances, trace widths, drill sizes, and unrouted connections.

- Verified minimum copper-to-copper clearance was respected across all nets

- Confirmed all nets were fully routed with no remaining ratsnest connections

- Checked drill sizes and annular rings against standard fabrication tolerances

- Verified footprint courtyard overlaps were resolved

Result: No outstanding DRC violations.

# 6. Fabrication Outputs

Once the design passed both ERC and DRC, the following manufacturing files were generated directly from KiCad for submission to a PCB fabrication and assembly house.

| Output | Format | Purpose |
| --- | --- | --- |
| Gerber files | RS-274X (.gbr) | Defines copper layers, silkscreen, soldermask, and board outline for fabrication |
| Drill file | Excellon (.drl) | Defines hole positions and sizes for through-hole components and vias |
| Bill of Materials | CSV | Lists all components, quantities, and reference designators for procurement/assembly |
| Pick-and-place file | CSV | Component placement coordinates and orientation for automated assembly |

## 6.1 Bill of Materials (Summary)

| Ref | Component | Value / Part No. | Qty |
| --- | --- | --- | --- |
| U1 | Switching Regulator | LM2596S-5.0 | 1 |
| L1 | Power Inductor | 68 µH | 1 |
| D1 | Schottky Diode | 1N5822 | 1 |
| C1 | Electrolytic Capacitor | 100 µF / 25V | 1 |
| C2 | Low-ESR Electrolytic Capacitor | 220 µF / 25V | 1 |
| J1 | Terminal Block (Input) | 2-pin, 5.08mm pitch | 1 |
| J2 | Terminal Block (Output) | 2-pin, 5.08mm pitch | 1 |

A full BOM including manufacturer part numbers, suppliers, and footprints is provided as a separate CSV file alongside this report.

# 7. Conclusion

This project successfully delivered a complete design package for a 12V-to-5V buck converter, covering schematic capture, datasheet-driven component selection, 2-layer PCB layout following power-electronics best practices, full electrical and physical design validation, and generation of a manufacturer-ready fabrication package.

The design demonstrates a practical, end-to-end understanding of switch-mode power supply design and the PCB design workflow using industry-standard EDA tooling (KiCad), from initial concept through to files ready for fabrication and assembly.

## 7.1 Future Improvements

- Add a dedicated test point header for bench measurement of output voltage, ripple, and load current

- Evaluate a synchronous buck topology to improve efficiency at lighter loads

- Add input reverse-polarity and overcurrent protection circuitry

- Develop a compact SMD variant of the same circuit for size-constrained applications

## 7.2 References

Texas Instruments, LM2596 SIMPLE SWITCHER Power Converter 150-kHz 3A Step-Down Voltage Regulator, Datasheet.

KiCad EDA Documentation, https://docs.kicad.org